Checklist for ensuring compliance of the Ex-product with relevant certificate

Equipment type:

Product certificate number:

Date of control:                                                                         Name of controller:

Note: for general check points, where “if any” is mentioned, the controller should check if it is applicable for the product under evaluation or not.

For the Check points relevant to specific type of protection(s), all of the checkpoints may be applicable or not applicable to the product under evaluation.

| # | Check point | Control method | Stage of control | Control period | Control result |  | Reference documents |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | accept | Reject |  |
| General check points |  |  |  |  |  |  |  |
| 1 | equipment type(s) and model(s) | Compliance with Certificate (Product description) | Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 2 | ATEX marking(s) | Compliance with Certificate (Marking) and TF Maring & Label part | Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 3 | warning marking(s) | Compliance with Certificate (Warning part) | Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 4 | used components in the equipment | Compliance with TF and Certificate (special condition of use) | Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 5 | validity of ATEX certificate | 1. Checking the expire date of certificate (if there is any expire date on certificate) 2. In cases of existence expiry date on certificate, check the status of validity in the NB website. | in each control period regardless the stage of product realization | Annual / product modification / standards revision |  |  |  |
| 6 | Metallic materials, if any | Compliance with TF and Certificate (product description) | 1. supplier initial selection and new suppliers 2. incoming material quality control | Sufficient numbers of sample in each produced batch |  |  |  |
| 7 | Non-metallic materials, if any |  |  |  |  |  |  |
| 8 | Painting (thickness), if any | Compliance with TF | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 9 | Fasteners, if any | Compliance with TF and Certificate (product description) | 1. supplier initial selection and new suppliers 2. incoming material quality control | Sufficient numbers of sample in each produced batch |  |  |  |
| 10 | Mounting of Ex-components, if any | Compliance with TF and Schedule of limitation of related Certificates | Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 11 | Cables characteristics (Size, type, temperature), if any | Compliance with TF and Certificate (product description) | 1. supplier initial selection and new suppliers 2. incoming material quality control | Sufficient numbers of sample in each produced batch |  |  |  |
| 12 | Overall dimensions | Compliance with schedule drawing | Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 13 | Product modifications | Check if the applied modification(s) have been communicated to the relevant NB, reviewed and evaluated and approved by the NB. | in each control period | in each control period |  |  |  |
| Check points relevant to specific type of protection(s) |  |  |  |  |  |  |  |
| Equipment with Flameproof Enclosure (Type of protection “Ex db”) |  |  |  |  |  |  |  |
| 1 | Non-threaded Flameproof joints Dimensions, tolerances | Compliance with Drawings | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 2 | Threaded Flameproof joints Dimensions, tolerances | Compliance with Drawings | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 3 | Flameproof joint machining, roughness and coating | Compliance with Drawings | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 4 | Cemented joints: Distance, visual inspection | Compliance with Drawings | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 5 | Bearings | Compliance with TF | 1. supplier initial selection and new suppliers 2. incoming material quality control | Sufficient numbers of sample in each produced batch |  |  |  |
| 6 | Special fasteners | Compliance with TF and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control | Sufficient numbers of sample in each produced batch |  |  |  |
| 7 | Entries for flameproof enclosure Dimension and tolerance check | Compliance with Drawing, Visual inspection and test | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 8 | Routine testing | Performing routine test according to certificate | Final equipment inspection and control | 100% of manufactured products |  |  |  |
| Increased Safety equipment (Type of protection “Ex eb”, “Ex ec”) |  |  |  |  |  |  |  |
| 1 | Terminals dimension | Compliance with TF and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control 3. Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 2 | Conductors (type, size, material, temperature, …) | Compliance with TF and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control 3. Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 3 | Connection of conductors in connection facilities | Compliance with Drawing and TF | Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 4 | Ex component (Ex-Certified terminals) | Compliance with TF and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control 3. Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 5 | Clearance & Creepage distance | Compliance with Drawing | Final equipment inspection and control | Sufficient numbers of sample in each produced batch |  |  |  |
| 6 | PCB conformal coating condition (effective, lasting, unbroken) | Compliance with Drawing and Certificate (Specific condition of use) | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 7 | Connection and impregnation of winding RTDs | Compliance with Drawing and TF | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 8 | Internal wiring (mechanically protected, secured, or routed to avoid insulation damage) | Compliance with Drawing and TF | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 9 | Electrical machines Case and winding dimension- air gap | Compliance with Drawing and TF | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 10 | Cell or battery internal connections | Compliance with Drawing and TF | During production (after related process) | Sufficient numbers of sample in each produced batch |  |  |  |
| 11 | Routine testing | Performing routine test according to certificate | Final equipment inspection and control | 100% of manufactured products |  |  |  |
| Intrinsically safe equipment (Type of protection “Ex ia”, “Ex ib”, “Ex ic”) |  |  |  |  |  |  |  |
| 1 | Internal wiring (material, size, …) | Compliance with Drawing and TF | 1. supplier initial selection and new suppliers 2. incoming material quality control |  |  |  |  |
| 2 | PCB (thickness, material, …) | Compliance with Drawing and TF | 1. supplier initial selection and new suppliers 2. incoming material quality control |  |  |  |  |
| 3 | Terminals dimension | Compliance with TF and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control 3. Final equipment inspection and control |  |  |  |  |
| 4 | Clearance & Creepage distance | Compliance with Drawing | Final equipment inspection and control |  |  |  |  |
| 5 | Coating, encapsulation, insulation materials | Compliance with Drawing and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control |  |  |  |  |
| 6 | Rating of components on which intrinsic safety depends | Compliance with Drawing and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control |  |  |  |  |
| 7 | Routine testing | Performing routine test according to certificate | Final equipment inspection and control |  |  |  |  |
| Encapsulated equipment (Type of protection “Ex m”) |  |  |  |  |  |  |  |
| 1 | Compound material | Compliance with Drawing and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control |  |  |  |  |
| 2 | Applying encapsulation condition (effective, lasting, unbroken) | Compliance with Drawing and Certificate (Specific condition of use) | During production (after related process) |  |  |  |  |
| 3 | Minimum thickness of compound | Compliance with Drawing and Certificate (Specific condition of use) | During production (after related process) |  |  |  |  |
| 4 | Protective devices | Compliance with TF and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control |  |  |  |  |
| 5 | Routine testing | Performing routine test according to certificate | Final equipment inspection and control |  |  |  |  |
| Equipment dust ignition protection by enclosure (Type of protection “Ex t”) |  |  |  |  |  |  |  |
| 1 | Protective devices | Compliance with TF and Certificate (Specific condition of use) | 1. supplier initial selection and new suppliers 2. incoming material quality control |  |  |  |  |
| 2 | Threaded joints, tolerances, quality, number of turns | Compliance with Drawings | During production (after related process) |  |  |  |  |
| 3 | Entries for enclosure Dimension and tolerance check | Compliance with Drawing, Visual inspection and test | During production (after related process) |  |  |  |  |