Checklist for verification compliance of the supplied product with relevant Ex final product certificate

Equipment/component to be supplied:                                         relevant supplier’s name:

relevant Ex final product type:                                                            Product certificate number:

Date of control:                                                                                            Name of controller:

Note:

- The below controls should be done on the equipment/component/material to be supplied;

- Supplied “material” consist

  - raw material (e.g. steel plate, gasket, Oring, compound and soon) and

  - parts (e.g. fastener, screw, cable, nut, bearing and so on)

- The manufacturer of the final product (purchaser of the supplied equipment/component) is responsible for implementation below controls;

- for general check points, where “if any” is mentioned, the controller should check if it is applicable for the product under evaluation or not.

- For the Check points relevant to specific type of protection(s), all of the checkpoints may be applicable or not applicable to the product under evaluation.

- Check points relevant to specific type of protection(s), should be completed only in cases that the supplied equipment or component are previously certified for ATEX or IECEx.

| # | Check point for the supplied equipment/component | Control method | Stage of control | Control period / number to be controlled | Control result |  | Reference documents |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | accept | Reject |  |
| General checkpoints |  |  |  |  |  |  |  |
| 1 | Equipment/component/material type(s) and model(s) | Compliance with the specification of components described in technical file and Ex product certificate | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 2 | ATEX marking(s) | Compliance the marking in the label of the component, with the specified Ex-marking of components in technical file and Ex product certificate | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 3 | warning marking(s) | Compliance the marking in the label of the component, with the specified Ex-marking of components in technical file and Ex product certificate | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 4 | validity of ATEX certificate | in the certificate of components: 1. Checking the expire date of certificate (if there is any expire date on certificate) 2. In cases of existence expiry date on certificate, check the status of validity in the NB website. | For each supplied batch | For each supplied batch |  |  |  |
| 5 | Metallic materials | Compliance with the mentioned information on technical file and Certificate of the final product Note: according to part j) of clause 8.4.2 of EN ISO/IEC 80079-34:2020, a specific analysis certificate or declaration shall be Presented and the reference number of it, should be mentioned in this report. | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 6 | Non-metallic materials |  |  |  |  |  |  |
| 7 | Painting (thickness) | Compliance with the mentioned information on technical file and Certificate of the final product | After finishing the painting process by supplier | Sufficient numbers of sample in each supplied batch |  |  |  |
| 8 | Fasteners | Compliance with the mentioned information on technical file and Certificate of the final product | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 9 | Cables characteristics (Size, type, temperature) | Compliance with the mentioned information on technical file for and Certificate of the final product (the part of product description) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 10 | Overall dimensions | Compliance with the mentioned information on technical file and Certificate of the final product (schedule drawings) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 11 | Supplied component modifications | Check if the applied modification(s) on supplied components have been reviewed and communicated to the relevant NB, and evaluated and approved by the NB. | in each control period | in each control period |  |  |  |
| Below checks should be done only in case that the possibility for conducting self-control by the supplier has been previously checked by the manufacturer and approved. |  |  |  |  |  |  |  |
| 12 | Provided inspection report by the supplier, Only in case that the supplier has been previously authorized by the manufacturer to perform independent inspection and test according to part b) of cluse 8.5.2 of EN ISO/IEC 80079-34:2020 | Check if the inspection report covers - all of the specifications in incoming material control plan and also - applicable check points of this checklist | Receiving an incoming batch | Each incoming batch |  |  |  |
| 13 | Declaration of conformity (DOC) provided by the supplier | 1- Existence documented objective evidence for supplier evaluation demonstrating the supplier is fully capable of producing and verifying the process, product or service; Reference to Point c) of 8.4.2 of EN ISO/IEC 80079-34:2020 2- complying the content of DOC with the criteria defined in https://conformex.it/Atex/Detail/EU-DOC-ATEX | Receiving an incoming batch | Each incoming batch |  |  |  |
| Check points relevant to specific type of protection(s) * This part should be completed only in cases that the supplied equipment or component has not been previously certified for ATEX or IECEx. |  |  |  |  |  |  |  |
| Equipment with Flameproof Enclosure (Type of protection “Ex db”) |  |  |  |  |  |  |  |
| 1 | Non-threaded Flameproof joints Dimensions, tolerances | Compliance of the supplied component with schedule and related drawings | During supplier production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 2 | Threaded Flameproof joints Dimensions, tolerances | Compliance of the supplied component with schedule and related drawings | During supplier production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 3 | Flameproof joint machining, roughness and coating | Compliance of the supplied component with schedule and related drawings | During supplier production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 4 | Cemented joints: Distance, visual inspection | Compliance of the supplied component with schedule and related drawings | During supplier production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 5 | Bearings | Compliance with TF | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 6 | Special fasteners | Compliance with TF and Certificate (Specific condition of use) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 7 | Entries for flameproof enclosure Dimension and tolerance check | Compliance with Drawing, Visual inspection and qualitative control by go-not go gauge | During supplier production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 8 | Routine testing for completed supplied components | Performing routine test according to certificate | Checking the reports issued by supplier on the basis of performed test in supplier location | 100% of components before shipment from supplier location |  |  |  |
| Increased Safety equipment (Type of protection “Ex eb”, “Ex ec”) |  |  |  |  |  |  |  |
| 1 | Terminals dimension | Compliance with TF and Certificate (Specific condition of use) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 2 | Conductors (type, size, material, temperature, …) | Compliance with TF and Certificate (Specific condition of use) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 3 | Connection of conductors in connection facilities | Compliance with Drawing and TF | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 4 | Clearance & Creepage distance | Compliance with Drawing | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 5 | PCB conformal coating condition (effective, lasting, unbroken) | Compliance with Drawing and Certificate (Specific condition of use) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 6 | Connection and impregnation of winding RTDs | Compliance with Drawing and TF | During supplier production (during related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 7 | Internal wiring (mechanically protected, secured, or routed to avoid insulation damage) | Compliance with Drawing and TF | During production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 8 | Electrical machines: Case and winding dimension- air gap | Compliance with Drawing and TF | During production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 9 | Cell or battery internal connections | Compliance with Drawing and TF | During production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 10 | Routine testing | Performing routine test according to certificate | Checking the reports issued by supplier on the basis of performed test in supplier location | 100% of components before shipment from supplier location |  |  |  |
| Intrinsically safe equipment (Type of protection “Ex ia”, “Ex ib”, “Ex ic”) |  |  |  |  |  |  |  |
| 1 | Internal wiring (material, size, …) | Compliance with Drawing and TF | During production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 2 | PCB (thickness, material, …) | Compliance with Drawing and TF | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 3 | Terminals dimension | Compliance with TF and Certificate (Specific condition of use) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 4 | Clearance & Creepage distance | Compliance with Drawing | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 5 | Coating, encapsulation, insulation materials | Compliance with Drawing and Certificate (Specific condition of use) | During production (during related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 6 | Rating of components on which intrinsic safety depends | Compliance with Drawing and Certificate (Specific condition of use) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 7 | Routine testing | Performing routine test according to certificate | Checking the reports issued by supplier on the basis of performed test in supplier location | 100% of components before shipment from supplier location |  |  |  |
| Encapsulated equipment (Type of protection “Ex m”) |  |  |  |  |  |  |  |
| 1 | Compound material | Compliance with TF | During production (during related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 2 | Applying encapsulation condition (effective, lasting, unbroken) | Compliance with TF | During production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 3 | Minimum thickness of compound | Compliance with TF | During production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 4 | Protective devices | Compliance with TF and drawings | During production (during related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 5 | Routine testing | Performing routine test according to certificate | Checking the reports issued by supplier on the basis of performed test in supplier location | 100% of components before shipment from supplier location |  |  |  |
| Equipment dust ignition protection by enclosure (Type of protection “Ex t”) |  |  |  |  |  |  |  |
| 1 | Protective devices | Compliance with TF and Certificate (Specific condition of use) | Incoming material control | Sufficient numbers of sample in each supplied batch |  |  |  |
| 2 | Threaded joints, tolerances, quality, number of turns | Compliance of the supplied component with schedule and related drawings | During supplier production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |
| 3 | Entries for enclosure Dimension and tolerance check | Compliance with Drawing, Visual inspection and qualitative control by go-not go gauge | During supplier production (after related process) | Sufficient numbers of sample in each supplied batch |  |  |  |